Faculty Location Monitoring System Using RFID Classroom Check-In

Submitted by:

Fabito, John Carlo

Flores, Brendon

Mangubat, Jenny

Patnon, Mikaela F.

May 2026

Table of Contents

Project Overview	3

Project Description	3

Problem Statement	3

Objectives	3

General Objective	3

Scope and Limitations	4

Scope of the Study	4

Limitations of the Study	4

System Architecture	5

Architecture Description	5

Data Flow	6

Hardware Components	7

Software Components	7

Functional Requirements	7

Non-Functional Requirements	8

Database Design (Sample Fields)	9

System Operation Procedure	9

Gantt Chart	10

Expected Output	12

Development Period	12

Proof of System Functionality	13

Source Code	14

# Project Overview

# Project Description

This system utilizes passive or active RFID tags embedded in faculty ID cards or carried as key fobs, which are read by RFID scanners installed at the entrance of each classroom. When a faculty member enters a classroom, the reader identifies them, captures the timestamp, and transmits this data to a central database via a microcontroller. The system automatically saves the time, date, and location in a centralized database, allowing administrators to monitor attendance in real time. This reduces paperwork, prevents errors, and improves accountability within the campus. Recent studies on RFID-based attendance systems show that this technology significantly improves efficiency and minimizes time consumption compared to manual methods.

According to Klaus Finkenzeller (2010), RFID technology enables automatic identification and efficient data collection without the need for manual input. RFID systems are composed of tags, readers, and backend databases that work together to ensure fast and accurate information processing. This supports the proposed system by demonstrating how RFID can streamline attendance tracking and reduce administrative workload in educational institutions.

The integration of RFID technology in the proposed system provides a modern solution for faculty monitoring by ensuring real-time data capture, improved accuracy, and efficient management of attendance records.

# Problem Statement

Manual attendance systems rely a lot on people to input data, which makes them prone to mistakes, inconsistencies and inefficiencies. When faculty members record attendance they often do it in a rush, which can lead to incomplete entries. Problems like someone recording attendance for another person missed records and inconsistent documentation make attendance data unreliable.

Here are the specific problems this study looks at:

1. Manual attendance recording is a waste of time

Traditional attendance methods, including paper-based logs and manual sign-in sheets, are time-consuming and disrupt the start of classes. These processes reduce valuable instructional time and create administrative delays, particularly in large institutions.

2. No real-time data and monitoring

Current systems do not give access to attendance data. This means administrators have to wait to find out who is absent late or not where they are supposed to be, which limits their ability to make decisions and supervise classrooms effectively.

3. Inaccuracies and security issues

Manual systems are prone to mistakes like incorrect data entry and lost records. They are also vulnerable to cheating like someone recording attendance for another person with no way to verify if the person is actually there.

4. Limited tracking of faculty location

This methods only confirm if a faculty member has recorded attendance but do not check where they are on campus or if they are in the right classroom during scheduled times.

# Objectives

# General Objective

To develop an IoT-based system that automates classroom attendance recording and monitoring.

Specific Objectives

The study aims to achieve the following specific objectives:

- To eliminate time wasted during manual roll calls by enabling instantaneous recording of faculty attendance via RFID card tapping.

- To develop a centralized database system that stores "in" and "out" time-stamps in real-time, preventing fraudulent "buddy-punching" or proxy attendance.

- To enable administrators to monitor the presence or absence of faculty in specific locations (e.g., in a lecture hall or laboratory) to ensure class schedules are followed.

- To increase the reliability of location tracking and attendance records by reducing human error.

- To create a system that can generate daily, weekly, or monthly attendance reports for HR or administrative analysis.

- To develop a system that can be integrated with existing university information systems or ERP software.

# Scope and Limitations

# Scope of the Study

This project focuses on developing a Faculty Location Monitoring System that uses RFID for Classroom Check-In.

The specific goals of this study include the following:

• The system uses RFID technology to track faculty classroom check-in.

• Each faculty member receives a unique RFID for identification.

• An RFID reader is set up at designated classroom entrances.

• The system records the faculty ID, name, classroom location, date, and time of check-in.

• A centralized database keeps attendance and location records.

• An administrative interface allows authorized users to view logs and create reports.

• The system is designed only for use on school campuses.

• The project shows basic automation in tracking attendance and location.

# Limitations of the Study

Even with its intended features, the proposed system has some limitations that need to be mentioned.

These limitations include the following:

• The system is not built for continuous real-time tracking across the whole campus.

• It only records data when a faculty member scans their RFID.

• If a faculty member fails to scan the RFID, attendance will not be recorded.

• The system cannot automatically detect when a faculty member leaves the classroom unless a scan-out feature is added.

• The system only works in classrooms with RFID readers.

• Technical issues like power outages, hardware malfunctions, or network problems may affect performance.

• The system is meant only for monitoring faculty and does not track student attendance.

# System Architecture

Figure 1: System Architecture

# Architecture Description

The system architecture illustrates the interaction between RFID hardware, an ESP32 microcontroller, LoRa transceivers, and a cemralized monitoring interface. The design follows a distributed lot architecture, whore RFID data is captured locally, transmitted via Lola, and aggregated at a gateway for storage and visualization. This minimizes wiring complexity across buildings and leverages LoRa's long-range communication for reliable faculty tracking

1. RFID Data Acquisition

The process begins when a faculty member taps their RFID card on the RC522 reader located at the Faculty Building entrance. Each RFID tag contains a unique identifier (UID). The RFID reader captures this UID and forwards it to the ESP32 microcontroller va SPi communication. This ensures fast, contactless identification without manual input

2. Local Processing (ESP32)

The ESP32 receives the RFID UID and packages it with a timestamp. Basic validation (eg, checking against registered (Ds) can be performed locally. The ESP32 then prepares the data for transmission over Lofa

3. LoRa Transmission

The ESP32 communicates with the LoRa transceiver (SX1278/RFM95). The UID and timestamp an transmitted wirelessly to the Teaching Building, where the LoRa receiver is located. LoRa's long-range, low-power communication ensures reliable data transfer even across multiple buildings.

4. LoRa Reception and Gateway Processing

Al the Teaching Building, the LoRa receiver module forwards the received data to another ESP32. This ESP32 acts as a gateway, parsing the incoming data and preparing it for display and storage

5. Display and Monitoring

The gateway ESP32 drives a local LCOILED screen to show real-time faculty presence. Optionally, the gatewiary can forward data to a centralized database (MySQL or Firebase for long-term storage and web dashboard visualization

6.Centralized Database and Dashboard (Optional Extension)

For scalability, the gateway ESP32 can connect to a local server or cloud database. A web dashboard allows administrators to view logs, track faculty movement, and manage registered users. This provides a user friendly interface for monitoring attendance and presence across campus.

# Data Flow

Figure 2 Data Flow Diagram

The data flow of the Faculty Location Monitoring System Using RFID Classroom Check-In follows a structured and continuous process: RFID detection → data transmission → processing and validation → database storage → retrieval → web-based presentation. This organized flow ensures accurate faculty identification, reliable data storage, and real-time monitoring through a centralized web dashboard.

1. Data Input  (RFID Interaction)
The data flow begins when a faculty member taps their RFID card on the RFID reader installed in the classroom. Each RFID card contains a unique identifier (UID), which serves as the primary data used to identify the faculty member. The RFID reader captures this UID automatically, ensuring fast, contactless identification and eliminating manual data entry errors.

2. Data Transmission to the Microcontroller
After detecting the RFID card, the RFID reader sends the UID to the ESP32 microcontroller through a wired communication interface. At this stage, the data consist of the raw UID, which represents the identity of the faculty member but has not yet been processed or validated.

3. Data Processing and Preparation
The ESP32 microcontroller processes the received UID and attaches additional information such as the classroom ID and the current date and time. This creates a complete check-in record containing the necessary information for faculty location tracking. The ESP32 then prepares the data for transmission to the web server.

4. Data Transmission to the Web Server
The ESP32 sends the check-in data, including the UID, classroom ID, and timestamp, to the web server through a WiFi connection using HTTP or API communication. This step transfers the collected data from the hardware device to the centralized system for validation and storage.

5. Data Validation and Record Generation
Upon receiving the data, the web server validates the UID by comparing it with registered faculty records stored in the database. If the UID is recognized as a registered faculty member, the server generates a structured check-in record containing faculty identity, classroom location, and timestamp.

6. Database Storage
The validated check-in record is stored in the system database. The database maintains organized records of faculty information, RFID UID assignments, classroom details, and check-in logs. This ensures permanent storage and allows future retrieval for monitoring and reporting purposes.

7. Data Retrieval for Monitoring
When an administrator accesses the web dashboard, the web server retrieves the latest faculty check-in records from the database. The system ensures that all retrieved information reflects the most recent check-in activity for accurate monitoring.

8. Data Presentation and Continuous Update
The retrieved data are displayed on the web dashboard, allowing administrators to view faculty locations, check-in history, and attendance records in real time. The system operates continuously, and each new RFID scan automatically updates the database and dashboard, ensuring accurate and up-to-date faculty location monitoring.

# Hardware Components

| Component | Description | Minimum Requirements | Recommended Requirements | Price |
| --- | --- | --- | --- | --- |
| DS3231 RTC Module | Real-Time Clock for accurate timestamping of check-ins/check-outs | ESP8266 microcontroller | ESP32 |  |
| RFID Reader /Cards | Reads RFID card/faculty identification | (MFRC522) | (MFRC522) | P114 |
| ESP32 Microcontroller | Controls the system, interfaces with RFID, DS3231, LCD, LoRaWAN |  |  |  |
| LCD Display | Shows current check-in status or messages |  | I2cc iic | P1,079 |
| LoRaWAN Module | Wireless communication to central server (if system is distributed | Ra-02 433M |  | P1,659 |
| SD Card Module | Local storage of logs or backups |  |  |  |  |
| Antenna Aerial | Connection Wifi | 2.4Ghz 3dBi WiFi 2.4g | 2.4Ghz 3dBi WiFi 2.4g | P284 |  |
| Power Supply | Powers ESP32 and peripherals |  |  | P339 |  |
| Miscellaneous | Cables, connectors |  | Type C Cord |  |  |

Table 1: Hardware Components

# Software Components

| Software | Purpose | Minimum Requirements | Recommended Requirements |
| --- | --- | --- | --- |
| Arduino IDE | Firmware development | Windows 7/10, USB port | Windows 10/11, stable USB and internet connection |
| Firebase / MySQL | Cloud database | Internet connection, 1GB storage, basic hosting/server support | High-speed internet, scalable cloud plan, real-time database support, backup enabled |
| Javascript /Firebase | Backend API |  |  |
| Java Script | Frontend dashboard | Modern browser (Chrome/Firefox), basic HTML/CSS support | Latest browser version, frameworks (React/Vue), responsive UI design |
| Web Browser | System access | Google Chrome/Firefox, 2GB RAM device, internet connection | Latest Chrome/Edge, 4GB+ RAM device, high-speed internet for real-time updates |

Table 2: Software Components

# Functional Requirements

Functional requirements describe what the system should do. In the Faculty Location Monitoring System Using RFID Classroom Check-In, the system shall meet the following requirements:

1.Faculty Authentication via RFID

The system must read the RFID card of each faculty member. Each RFID card is linked to a unique faculty ID stored in the database to ensure accurate identification and prevent unauthorized access. This function enables fast, contactless verification of faculty presence.

2. Classroom Check-In / Check-Out

Faculty members can check in by tapping their RFID card on the classroom reader. The system logs the time and classroom location of each check-in. Optionally, the system can support check-out or automatic logout when a faculty member leaves the classroom, ensuring accurate real-time records of faculty presence.

3. Real-Time Location Tracking

The system displays which faculty members are currently present in each classroom and updates this information in real time on the web interface. This feature allows administrators to monitor classroom occupancy and faculty movement efficiently.

4. Attendance Record Management

The system automatically records attendance based on RFID check-ins. It can generate daily, weekly, or monthly attendance reports and allows authorized users to export these records in formats such as CSV or PDF. This ensures organized and accessible attendance management.

5. Admin Interface

Authorized personnel can add, edit, or remove faculty information, view location and attendance logs for auditing purposes, and assign classrooms to faculty members. The admin interface provides a centralized platform for managing the system efficiently.

# Non-Functional Requirements

Non-functional requirements define the quality attributes of the Faculty Location Monitoring System Using RFID Classroom Check-In and describe how well the system performs its functions rather than what functions it performs. The system shall meet the following requirements:

1.Performance

The system is required to record faculty check-ins within two (2) seconds after an RFID card is scanned. This ensures that the check-in process is fast enough to prevent delays during classroom entry and allows the system to handle multiple scans efficiently, especially at the start of classes.

2. Security

The system must implement authentication mechanisms for dashboard access to ensure that only authorized users, such as administrators, can view or manage faculty location and attendance records. This protects sensitive data from unauthorized access, ensures accountability, and supports compliance with data privacy policies.

3. Reliability

The system must maintain data integrity during transmission and storage. Check-in data captured from RFID scans should be accurately transmitted and stored without loss, duplication, or corruption. This ensures that faculty location and attendance records remain consistent and trustworthy, even during repeated operations or extended system use.

4. Scalability

The system must be capable of supporting multiple classrooms and an increasing number of faculty members without significant degradation in performance. This allows the system to accommodate additional RFID readers, users, and attendance records as the institution grows, ensuring future expansion is possible.

5. Usability

The system shall provide a user-friendly interface for administrators and authorized personnel, requiring minimal training to operate. Clear dashboards, intuitive navigation, and easy access to reports ensure efficient use of the system.

6.Maintainability

The system shall be designed for easy maintenance, updates, and troubleshooting. Software and hardware components should allow quick upgrades or replacement without disrupting ongoing operations.

7.Portability

The system shall be accessible from different devices, such as desktop computers, laptops, and tablets, ensuring that administrators can monitor faculty location and attendance from multiple locations without being confined to a single workstation.

# Database Design (Sample Fields)

| Field | Description |
| --- | --- |
| faculty_id | Unique faculty identifier |
| rfid_uid | RFID card UID |
| name | facultyname |
| timestamp | Date and time of attendance |
| roomNum | Room Number |

# System Operation Procedure

This section describes the step-by-step procedure for operating the Faculty Location Monitoring System Using RFID Classroom Check-In System. The procedure outlines the normal sequence of actions required to ensure proper system functionality from device startup to monitoring faculty location via the web dashboard.

Power On the RFID Classroom Device

Step 1:

The RFID classroom device is powered on using a stable power supply. Upon startup, the system initializes all hardware components, including the RFID reader, microcontroller indicator LEDs, buzzer, and network interface. This initialization process prepares the device for faculty check-in operations.

Establish Wi-Fi Connection

Step 2:

The system automatically attempts to connect to the configured Wi-Fi network. A successful connection enables communication between the classroom device and the central web server. Once connected, the device enters standby mode and waits for RFID card input.

Faculty Validation and Location Recording

Step 3:

The system verifies the scanned UID against the registered faculty database stored in the server.

Viewing Faculty Location via Web Dashboard

Step 4:

The administrator or authorized personnel accesses the web dashboard through a web browser within the authorized network environment. The dashboard displays faculty attendance and real-time classroom location records.

# Fabito, John Carlo

# Gantt Chart

| Task / Phase | Week 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Planning & Proposal | ███ | ███ |  |  |  |  |  |  |
| Requirements Analysis |  | ███ | ███ |  |  |  |  |  |
| System Architecture Design |  |  | ███ |  |  |  |  |  |
| Hardware Setup & Wiring |  |  |  | ███ |  |  |  |  |
| Firmware Development (ESP32) |  |  |  |  | ███ |  |  |  |
| RFID Integration |  |  |  |  | ███ | ███ |  |  |
| Web Server & Dashboard Development |  |  |  |  |  |  | ███ |  |
| Data Storage Configuration |  |  |  |  |  |  | ███ |  |
| System Integration |  |  |  |  |  |  | ███ |  |
| Testing & Debugging |  |  |  |  |  |  |  | ███ |
| Documentation Writing |  |  |  |  |  |  |  | ███ |
| Final Evaluation & Submission |  |  |  |  |  |  |  | ███ |

# Flores, Brendon

| Task / Phase | Week 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Planning & Proposal | ███ | ███ |  |  |  |  |  |  |
| Requirements Analysis |  | ███ | ███ |  |  |  |  |  |
| System Architecture Design |  |  | ███ |  |  |  |  |  |
| Hardware Setup & Wiring |  |  |  | ███ | ███ |  |  |  |
| Firmware Development (ESP32) |  |  |  |  | ███ |  |  |  |
| RFID Integration |  |  |  |  |  | ███ |  |  |
| Web Server & Dashboard Development |  |  |  |  |  |  | ███ |  |
| Data Storage Configuration |  |  |  |  |  |  | ███ |  |
| System Integration |  |  |  |  |  |  | ███ |  |
| Testing & Debugging |  |  |  |  |  |  |  | ███ |
| Documentation Writing |  |  |  |  |  |  |  | ███ |
| Final Evaluation & Submission |  |  |  |  |  |  |  | ███ |

# Mangubat, Jenny M.

| Task / Phase | Week 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Planning & Proposal | ███ | ███ |  |  |  |  |  |  |
| Requirements Analysis |  | ███ | ███ |  |  |  |  |  |
| System Architecture Design |  |  | ███ |  |  |  |  |  |
| Hardware Setup & Wiring |  |  |  | ███ | ███ |  |  |  |
| Firmware Development (ESP32) |  |  |  |  | ███ |  |  |  |
| RFID Integration |  |  |  |  |  | ███ |  |  |
| Web Server & Dashboard Development |  |  |  |  |  |  | ███ |  |
| Data Storage Configuration |  |  |  |  |  |  | ███ |  |
| System Integration |  |  |  |  |  |  | ███ |  |
| Testing & Debugging |  |  |  |  |  |  |  | ███ |
| Documentation Writing |  |  |  |  |  |  |  | ███ |
| Final Evaluation & Submission |  |  |  |  |  |  |  | ███ |

# Patnon, Mikaela F

| Task / Phase | Week 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Planning & Proposal | ███ | ███ |  |  |  |  |  |  |
| Requirements Analysis |  | ███ | ███ |  |  |  |  |  |
| System Architecture Design |  |  | ███ |  |  |  |  |  |
| Hardware Setup & Wiring |  |  |  | ███ | ███ |  |  |  |
| Firmware Development (ESP32) |  |  |  |  | ███ |  |  |  |
| RFID Integration |  |  |  |  |  | ███ |  |  |
| Web Server & Dashboard Development |  |  |  |  |  |  | ███ |  |
| Data Storage Configuration |  |  |  |  |  |  | ███ |  |
| System Integration |  |  |  |  |  |  | ███ |  |
| Testing & Debugging |  |  |  |  |  |  |  | ███ |
| Documentation Writing |  |  |  |  |  |  |  | ███ |
| Final Evaluation & Submission |  |  |  |  |  |  |  | ███ |

# Expected Output

The Faculty Location Monitoring System Using RFID Classroom Check-In is designed to deliver precise faculty tracking and attendance management. The expected outputs of the system are described as follows:

- RFID Card Reading and Validation
When a faculty member taps their RFID card on the classroom reader, the system shall read the unique identifier (UID) from the card, validate it against the registered faculty database, and confirm the authenticity of the user. This ensures only authorized faculty are recorded.

- Check-In Recording
The system shall record the date, time, and classroom location of each valid check-in. The captured information is immediately stored in a cloud-based database, ensuring centralized, persistent, and secure data management.

- Real-Time Location Display
The system shall provide a real-time view of faculty presence in each classroom through a web-based dashboard. Administrators can see which faculty members are currently in which classrooms, allowing efficient monitoring of classroom occupancy.

- Attendance Reports
The system shall generate attendance reports for daily, weekly, or monthly periods. Authorized users can access these reports via the dashboard and export them in formats such as CSV or PDF for record-keeping and analysis.

- Notifications and Alerts
The system shall notify administrators if a faculty member fails to check in or if an unauthorized RFID card is used. These alerts help maintain attendance compliance and enhance the security of the monitoring system.

- Dashboard Interface
The system shall provide a user-friendly dashboard interface that organizes and displays attendance logs, location data, and reports clearly. This ensures that administrators can easily navigate the system and make informed decisions without manual data handling.

# Development Period

Pictures During Development (Documentation Evidence)

Images provide visual documentation of the system’s development and implementation. Such documentation should be collected during the actual development process, not solely after project completion.

Figure 3: Development Period

Recommended Development Photos

- Hardware Assembly Photos

- Scanning Operation

- Web Dashboard Screenshots

- Testing and Debugging Activities

# Source Code

Gateway

#include <SPI.h>

#include <LoRa.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <WiFi.h>

#include <time.h>

#include <Firebase_ESP_Client.h>

#include "addons/TokenHelper.h"

#include "addons/RTDBHelper.h"

// ================= WIFI =================

#define WIFI_SSID "MBC_REGO WI-FI_5G"

#define WIFI_PASSWORD "xoxoxoxo"

// ================= FIREBASE =================

#define API_KEY "AIzaSyDH_licIPGRqWHHIGF4DEzJqeshH-gVPPc"

#define PROJECT_ID "flores-integ2f3-eef2b"

// ================= LORA =================

#define SS   27

#define RST  14

#define DIO0 26

// ================= BUTTON =================

#define BTN 32

// ================= LCD =================

LiquidCrystal_I2C lcd(0x27, 20, 4);

// ================= FIREBASE OBJECTS =================

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

bool signupOK = false;

// ================= STORAGE =================

String rooms[10];

String names[10];

int count = 0;

// ================= SCROLL =================

int displayIndex = 0;

bool lastBtnState = HIGH;

// ================= TIME =================

String getTimeNow() {

struct tm timeinfo;

if (!getLocalTime(&timeinfo)) return "NoTime";

char buffer[30];

strftime(buffer, sizeof(buffer), "%Y-%m-%d %H:%M:%S", &timeinfo);

return String(buffer);

}

void setup() {

Serial.begin(115200);

pinMode(BTN, INPUT_PULLUP);

lcd.init();

lcd.backlight();

lcd.print("Connecting WiFi");

// ===== WIFI =====

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

lcd.clear();

lcd.print("WiFi Connected");

// ===== FIREBASE =====

config.api_key = API_KEY;

if (Firebase.signUp(&config, &auth, "", "")) {

signupOK = true;

}

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

// ===== TIME =====

configTime(8 * 3600, 0, "pool.ntp.org", "time.nist.gov");

// ===== LORA =====

LoRa.setPins(SS, RST, DIO0);

if (!LoRa.begin(433E6)) {

lcd.clear();

lcd.print("LoRa FAILED!");

while (true);

}

lcd.clear();

lcd.print("Gateway Ready");

delay(2000);

lcd.clear();

}

void loop() {

// ================= RECEIVE LORA =================

int packetSize = LoRa.parsePacket();

if (packetSize) {

String incoming = "";

while (LoRa.available()) {

incoming += (char)LoRa.read();

}

Serial.println("Received: " + incoming);

int sep = incoming.indexOf('|');

if (sep != -1) {

String room = incoming.substring(0, sep);

String name = incoming.substring(sep + 1);

// ===== STORE =====

bool exists = false;

for (int i = 0; i < count; i++) {

if (rooms[i] == room) {

names[i] = name;

exists = true;

break;

}

}

if (!exists && count < 10) {

rooms[count] = room;

names[count] = name;

count++;

}

displayIndex = 0;

updateDisplay();

// ===== SEND TO FIREBASE =====

if (signupOK) {

FirebaseJson content;

content.set("fields/room/stringValue", room);

content.set("fields/name/stringValue", name);

content.set("fields/time/stringValue", getTimeNow());

String path = "rfid_logs/" + String(millis());

if (Firebase.Firestore.createDocument(&fbdo, PROJECT_ID, "", path.c_str(), content.raw())) {

Serial.println("✅ Sent to Firebase");

} else {

Serial.println("❌ Error: " + fbdo.errorReason());

}

}

}

}

// ================= BUTTON =================

bool btnState = digitalRead(BTN);

if (btnState == LOW && lastBtnState == HIGH) {

if (count > 0) {

displayIndex++;

if (displayIndex >= count) displayIndex = 0;

updateDisplay();

}

delay(200);

}

lastBtnState = btnState;

}

// ================= DISPLAY =================

void updateDisplay() {

lcd.clear();

for (int i = 0; i < 4; i++) {

int idx = displayIndex + i;

if (idx >= count) break;

lcd.setCursor(0, i);

lcd.print(rooms[idx]);

lcd.print(" ");

lcd.print(names[idx]);

}

}

Classroom 1

#include <SPI.h>

#include <MFRC522.h>

#include <LoRa.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// ================= RFID =================

#define SS_PIN 5

#define RST_PIN 4

MFRC522 mfrc522(SS_PIN, RST_PIN);

// ================= LORA =================

#define LORA_SS   27

#define LORA_RST  14

#define LORA_DIO0 26

// ================= LCD =================

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

Serial.begin(115200);

// LCD

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Starting...");

// RFID

SPI.begin();

mfrc522.PCD_Init();

// LoRa

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {

lcd.clear();

lcd.print("LoRa FAIL");

while (true);

}

lcd.clear();

lcd.print("Scan Card...");

}

void loop() {

// wait for RFID

if (!mfrc522.PICC_IsNewCardPresent()) return;

if (!mfrc522.PICC_ReadCardSerial()) return;

// ===== READ UID =====

String uid = "";

for (byte i = 0; i < mfrc522.uid.size; i++) {

uid += String(mfrc522.uid.uidByte[i], HEX);

}

uid.toUpperCase();

Serial.print("UID: ");

Serial.println(uid);

// ===== MAP UID → NAME + ROOM =====

String name = "";

String room = "";

if (uid == "46D3106") {

name = "C. Cabrera";

room = "ROOM1";

}

else if (uid == "2EABD26") {

name = "C. De Guzman";

room = "ROOM1";

}

else if (uid == "29C1136") {

name = "U. Millendrez";

room = "ROOM1";

}

else if (uid == "F12CF6") {

name = "N. Magnaye";

room = "ROOM1";

}

else if (uid == "E636D26") {

name = "G. Binay";

room = "ROOM1";

}

else if (uid == "4CC5106") {

name = "E. Rodriguez";

room = "ROOM1";

}

else {

name = "Unknown";

room = "ROOM?";

}

// ===== LCD DISPLAY =====

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(room);

lcd.setCursor(0, 1);

lcd.print(name);

// ===== LORA SEND =====

String message = room + "|" + name;

LoRa.beginPacket();

LoRa.print(message);

LoRa.endPacket();

Serial.println("Sent: " + message);

delay(3000);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Scan Card...");

}

Classroom 2

#include <SPI.h>

#include <MFRC522.h>

#include <LoRa.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// ================= RFID =================

#define SS_PIN 5

#define RST_PIN 4

MFRC522 mfrc522(SS_PIN, RST_PIN);

// ================= LORA =================

#define LORA_SS   27

#define LORA_RST  14

#define LORA_DIO0 26

// ================= LCD =================

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

Serial.begin(115200);

// LCD

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Starting...");

// RFID

SPI.begin();

mfrc522.PCD_Init();

// LoRa

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {

lcd.clear();

lcd.print("LoRa FAIL");

while (true);

}

lcd.clear();

lcd.print("Scan Card...");

}

void loop() {

// wait for RFID

if (!mfrc522.PICC_IsNewCardPresent()) return;

if (!mfrc522.PICC_ReadCardSerial()) return;

// ===== READ UID =====

String uid = "";

for (byte i = 0; i < mfrc522.uid.size; i++) {

uid += String(mfrc522.uid.uidByte[i], HEX);

}

uid.toUpperCase();

Serial.print("UID: ");

Serial.println(uid);

// ===== MAP UID → NAME + ROOM =====

String name = "";

String room = "";

if (uid == "46D3106") {

name = "C. Cabrera";

room = "ROOM2";

}

else if (uid == "2EABD26") {

name = "C. De Guzman";

room = "ROOM2";

}

else if (uid == "29C1136") {

name = "U. Millendrez";

room = "ROOM2";

}

else if (uid == "F12CF6") {

name = "N. Magnaye";

room = "ROOM2";

}

else if (uid == "E636D26") {

name = "G. Binay";

room = "ROOM2";

}

else if (uid == "4CC5106") {

name = "E. Rodriguez";

room = "ROOM2";

}

else {

name = "Unknown";

room = "ROOM?";

}

// ===== LCD DISPLAY =====

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(room);

lcd.setCursor(0, 1);

lcd.print(name);

// ===== LORA SEND =====

String message = room + "|" + name;

LoRa.beginPacket();

LoRa.print(message);

LoRa.endPacket();

Serial.println("Sent: " + message);

delay(3000);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Scan Card...");

}

Classroom 3

#include <SPI.h>

#include <MFRC522.h>

#include <LoRa.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// ================= RFID =================

#define SS_PIN 5

#define RST_PIN 4

MFRC522 mfrc522(SS_PIN, RST_PIN);

// ================= LORA =================

#define LORA_SS   27

#define LORA_RST  14

#define LORA_DIO0 26

// ================= LCD =================

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

Serial.begin(115200);

// LCD

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Starting...");

// RFID

SPI.begin();

mfrc522.PCD_Init();

// LoRa

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {

lcd.clear();

lcd.print("LoRa FAIL");

while (true);

}

lcd.clear();

lcd.print("Scan Card...");

}

void loop() {

// wait for RFID

if (!mfrc522.PICC_IsNewCardPresent()) return;

if (!mfrc522.PICC_ReadCardSerial()) return;

// ===== READ UID =====

String uid = "";

for (byte i = 0; i < mfrc522.uid.size; i++) {

uid += String(mfrc522.uid.uidByte[i], HEX);

}

uid.toUpperCase();

Serial.print("UID: ");

Serial.println(uid);

// ===== MAP UID → NAME + ROOM =====

String name = "";

String room = "";

if (uid == "46D3106") {

name = "C. Cabrera";

room = "ROOM3";

}

else if (uid == "2EABD26") {

name = "C. De Guzman";

room = "ROOM3";

}

else if (uid == "29C1136") {

name = "U. Millendrez";

room = "ROOM3";

}

else if (uid == "F12CF6") {

name = "N. Magnaye";

room = "ROOM3";

}

else if (uid == "E636D26") {

name = "G. Binay";

room = "ROOM3";

}

else if (uid == "4CC5106") {

name = "E. Rodriguez";

room = "ROOM3";

}

else {

name = "Unknown";

room = "ROOM?";

}

// ===== LCD DISPLAY =====

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(room);

lcd.setCursor(0, 1);

lcd.print(name);

// ===== LORA SEND =====

String message = room + "|" + name;

LoRa.beginPacket();

LoRa.print(message);

LoRa.endPacket();

Serial.println("Sent: " + message);

delay(3000);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("Scan Card...");

}