SYSTEM MANUAL

Faculty Location Monitoring System

Using RFID Classroom Check-In

USER GUIDE

Version 1.0  |  For Authorized Use Only

About This Manual

This manual provides clear operating instructions for the Faculty Location Monitoring System Using RFID Classroom Check-In. Since the hardware is already fully assembled and programmed, users only need to connect the power supply and follow the procedures in this guide.

The system consists of two main hardware units that work together:

| RFID Classroom Unit Reads faculty cards at the classroom entrance via RFID + LoRa | WIFI Gateway Unit Receives data via LoRa and uploads logs to Firebase via WiFi | CLOUD Dashboard Real-time browser-based monitoring at rfid.projectsmiledentalclinic.com |
| --- | --- | --- |

HOW IT WORKS A faculty member taps their RFID card at the classroom → the Classroom Unit reads it and sends the data via LoRa to the Gateway Unit → the Gateway uploads the log to Firebase → the Admin Dashboard updates in real time.

Both units must be powered on for the system to operate correctly.

1.  Powering On the System

Follow the steps below for each unit. Always power on the Classroom Unit first, then the Gateway Unit.

A.  Classroom Unit  (RFID Node)

| 1 | Connect the Power Supply Plug the Type-C USB cable from the Power Bank into the ESP32 Type-C port on the Classroom Unit. |
| --- | --- |

WHAT HAPPENS NEXT Once connected, the following sequence occurs automatically: 1. ESP32 powers on and boots up 2. RFID Reader (RC522) becomes active 3. LCD 20x4 display turns on 4. LoRa module begins transmitting

The Classroom Unit is now ready for faculty card scanning.

B.  Gateway Unit

| 1 | Connect the Power Supply Plug the Type-C USB cable from the Power Bank into the ESP32 Type-C port on the Gateway Unit. |
| --- | --- |

| 2 | Wait for WiFi Connection The Gateway automatically connects to the saved WiFi network. No manual configuration is needed. |
| --- | --- |

WHAT HAPPENS NEXT Once connected, the following sequence occurs automatically: 1. ESP32 Gateway turns on 2. LCD 20x4 display activates 3. LoRa receiver starts listening for classroom data 4. WiFi connects and Firebase link is established

WHY WIFI MATTERS The WiFi connection is required to upload attendance logs to Firebase Firestore, update the admin dashboard, and store faculty records in real time. Without WiFi, logs will not be saved to the cloud.

The Gateway Unit is now fully operational.

2.  Faculty Check-In Process

The following steps describe the complete check-in workflow — from card tap to cloud upload.

| 1 | Faculty Taps RFID Card The faculty member holds their issued RFID card near the RC522 reader at the classroom entrance. |
| --- | --- |

| 2 | System Reads the Card The RC522 reads the card's unique ID (UID). The ESP32 matches it to the registered faculty record. |
| --- | --- |

| 3 | LCD Confirms Check-In The 20x4 LCD screen displays the faculty name and assigned room, confirming successful check-in. |
| --- | --- |

LCD DISPLAY ROOM1 Prof. Juan Dela Cruz

| 4 | Data Sent to Gateway The Classroom Unit transmits the following data to the Gateway Unit via LoRa: |
| --- | --- |

| Faculty Name | Full name of the faculty member |
| --- | --- |
| Room Number | Assigned classroom identifier |
| Timestamp | Date and time of the check-in event |
| Attendance Log | Status record stored per session |

3.  Gateway System Operation

After check-in data is received from the Classroom Unit, the Gateway processes and displays the logs, then automatically syncs them to the cloud.

| 1 | Gateway Receives Faculty Logs All classroom check-in data arrives at the Gateway Unit via LoRa. Logs from multiple classrooms are aggregated and shown on the 20x4 LCD. |
| --- | --- |

LCD DISPLAY Prof: Juan Dela Cruz ROOM1 Prof: Maria Santos ROOM2 Prof: Pedro Reyes ROOM3

| 2 | Scroll Through Logs Press the Push Button to cycle through faculty records one at a time when multiple entries are present. |
| --- | --- |

SCROLL BUTTON Each press of the Push Button advances the display to the next faculty log entry. This allows staff to quickly review all active classroom check-ins.

| 3 | Automatic Firebase Upload The Gateway automatically sends all received logs to Firebase Firestore over WiFi. No manual action is required. |
| --- | --- |

CLOUD SYNC BENEFITS Real-time attendance recording — records update instantly Centralized cloud storage — accessible from any browser Dashboard monitoring — admin sees live data without delay

4.  Admin Dashboard Access

The admin dashboard is a web-based interface for monitoring all faculty check-ins in real time. It can be accessed from any device with a supported browser.

| 1 | Open a Supported Browser Launch any of the following browsers on your device: |
| --- | --- |

| Google Chrome | Microsoft Edge | Mozilla Firefox |
| --- | --- | --- |

| 2 | Go to the Dashboard URL Enter the following address in the browser's address bar and press Enter: |
| --- | --- |

https://rfid.projectsmiledentalclinic.com

| 3 | Sign In with Google Click Sign In and authenticate using your authorized Google account. Access is restricted to registered admin users only. |
| --- | --- |

| 4 | Monitor the Dashboard Once logged in, the dashboard displays: |
| --- | --- |

- Live classroom location of each faculty member

- Real-time check-in and check-out logs

- Attendance history per faculty and per room

5.  Troubleshooting Guide

Use this table to quickly identify and resolve common issues. If a problem persists after following the recommended steps, contact the system administrator.

| Problem | Recommended Solution |
| --- | --- |
| - No Power / Unit Not Starting | - Check that the Type-C cable is securely connected - Verify the power bank is turned on - Check the power bank battery level and recharge if low |
| - RFID Card Not Detected | - Confirm the RFID reader is powered on - Ensure you are using the correct registered card - Tap the card closer and flat against the reader surface |
| - LCD Not Displaying | - Check the power connection to the display - Verify all LCD wiring is secure - Restart the unit by disconnecting and reconnecting power |
| - Gateway Not Receiving Data | - Verify both units are powered on and fully booted - Check LoRa module connections on both units - Ensure both LoRa modules are set to the same frequency |
| - Firebase Not Updating | - Confirm the Gateway Unit has a stable WiFi connection - Restart the Gateway Unit - Verify internet access is available on the network |
| - Scroll Button Not Working | - Check the push button wiring at GPIO 32 - Restart the Gateway Unit - Replace the push button if wiring is confirmed correct |

6.  Important Reminders

Strict adherence to the following guidelines ensures reliable system performance, hardware longevity, and data integrity.

CHECK Always keep both the Classroom Unit and Gateway Unit powered on during operating hours.

CHECK Maintain a stable and active WiFi connection at the Gateway Unit at all times.

CHECK Keep all power banks fully charged before the start of each day.

CHECK Do not disconnect any unit while an active scanning or upload process is in progress.

CHECK Secure all hardware properly inside the project enclosure using the provided lock and key.

CRITICAL WARNING Never disconnect power during an active Firebase upload. Doing so may result in incomplete or corrupted records that cannot be recovered.